# Level 1: Emergency Hatch Authorization Recovery

AIM:

Following the crash, the primary access hatch has entered emergency lockdown. It only unlocks when the binary authorization sequence 1011 is received from the primary CPU, but the detection circuit has been damaged.

Design and implement an FSM-based sequence detector using D flip-flops that continuously monitors an incoming serial data stream. The detector must support overlapping sequence detection, so that partially overlapping transmissions are still recognized.

Each time the sequence is detected, an unlock signal is generated. For demonstration, wire this unlock signal to a 4-bit counter that tracks the total number of detected sequences.

Validation:

For the input stream 1 0 1 1 0 1 1 0 0 1 0 1 1 0, the counter should read 0011 (3 detections). Generate additional test inputs of your own and verify the counter output against your expected results.

# Approach:

Since the input bits arrive one at a time, the circuit needs memory to remember how much of the sequence has already been matched.

A Mealy FSM approach is used because the output depends on both the present state and the current input.

The sequence to be detected is: 1011

The FSM was divided into four states:

S0        - No sequence match

S1        - 1 has matched

S2        - 10 has Matched

S3        - 101 has Matched

When the FSM is in state S3 and the next input is 1, the full sequence 1011 is detected and the unlock output becomes high.

# State Transition Table:

Let the serial input be X.

| Present State | Input X | Next State | Match |
| --- | --- | --- | --- |
| S0 | 0 | S0 | 0 |
| S0 | 1 | S1 | 0 |
| S1 | 0 | S2 | 0 |
| S1 | 1 | S1 | 0 |
| S2 | 0 | S0 | 0 |
| S2 | 1 | S3 | 0 |
| S3 | 0 | S2 | 0 |
| S3 | 1 | S1 | 1 |

The important transition is:

The FSM goes to S1 after detection instead of S0 because the last received bit is 1, which can also act as the starting bit of another 1011 sequence. This is overlapping detection.

# State Encoding:

Since there are four states, two D flip-flops are required.

The state encoding used is:

| State | Q1 | Q0 |
| --- | --- | --- |
| S0 | 0 | 0 |
| S1 | 0 | 1 |
| S2 | 1 | 0 |
| S3 | 1 | 1 |

Here, Q0 and Q1 are the outputs of the two D flip-flops.

## D Flip-Flop Logic

| Present State | Present Q0 | Present Q1 | Input X | Next State | Next Q1 (D1) | Next Q0 (D0) | Match |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S0 | 0 | 0 | 0 | S0 | 0 | 0 | 0 |
| S0 | 0 | 0 | 1 | S1 | 0 | 1 | 0 |
| S1 | 1 | 0 | 0 | S2 | 1 | 0 | 0 |
| S1 | 1 | 0 | 1 | S1 | 0 | 1 | 0 |
| S2 | 0 | 1 | 0 | S0 | 0 | 0 | 0 |
| S2 | 0 | 1 | 1 | S3 | 1 | 1 | 0 |
| S3 | 1 | 1 | 0 | S2 | 1 | 0 | 0 |
| S3 | 1 | 1 | 1 | S1 | 0 | 1 | 1 |

For a D flip-flop:

Next value of Q = D, So,

next value of Q1 = D1

next value of Q0 = D0

### D0:                                                                         D1:

| Present Q0 | Present Q1 | Input X | Next Q0 (D0) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 |

| Present Q0 | Present Q1 | Input X | Next Q1 (D1) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 0 |

D0 = X                                                             D1 = Q0 X' + Q0' Q1 X

### Match:

| Present Q0 | Present Q1 | Input X | Match |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 |

Match = Q1 Q0 X

# Counter:

A 4-bit counter is used to count the number of detections.

Each detection pulse increases the counter by 1:

For the given input stream (10110110010110), the expected number of detections is 3, so the final counter output should be:

0011

# Test Cases:

## Test Case 1: Given Input Stream

Input Stream: 10110110010110

Output Expected: 3

Output: 0011 (3)

## Test Case 2: Single Detection

Input Stream: 1011

Output Expected: 1

Output: 0001 (1)

## Test Case 3:

Input Stream: 1011011

Output Expected: 2

Output: 0010 (2)

## Test Case 4: No Valid Sequence

Input Stream: 1111111

Output Expected: 0

Output: 0000 (0)

# Scope of Development:

This project can be further developed in the following ways:

- Programmable Sequence Detector

- Alarm System for Wrong Sequences

- Display Module:
A 7-segment display or LCD display